The pulp and paper industry is one of the largest industrial sectors in terms of water consumption and effluent discharge. Effluent from paper mills contains a complex mixture of contaminants including high chemical oxygen demand (COD), biochemical oxygen demand (BOD), suspended solids, lignin derivatives, and toxic chlorinated organic compounds such as adsorbable organic halides (AOX). Among these, color and AOX are the most persistent and difficult to treat. This article evaluates common and advanced neutralizing agents and water treatment strategies, including alum, lime, hydrogen peroxide, ozone, and Fenton’s reagent, with a comparative analysis of their effectiveness. Research-based insights highlight the promising role of advanced oxidation processes (AOPs) in addressing refractory pollutants in paper mill effluents.

Effluent treatment in paper mills is a critical environmental and regulatory issue. The production of paper generates large volumes of wastewater, often dark-colored and loaded with organic and inorganic pollutants. Traditional treatment methods (primary, secondary, and tertiary) are increasingly challenged by stricter environmental standards, particularly regarding color, COD, and AOX levels [1].

This article investigates the nature of paper mill effluents, identifies the most difficult contaminants, and compares the performance of various neutralizing and oxidizing agents in advanced effluent treatment systems.

## 2. Composition of Paper Mill Effluent

Paper mill wastewater can vary depending on the pulping process (kraft, mechanical, or chemical), but commonly includes:

- High BOD/COD: From organic waste such as lignin, cellulose fibers, and sugars.

- Color: Due to lignin degradation products.

- Suspended solids: From fiber loss and fillers.

- Toxic organics: Especially AOX from chlorine-based bleaching.

- Nutrients: Nitrogen and phosphorus, especially in coated paper production.

Among these, color (from lignin derivatives) and AOX are the most recalcitrant contaminants, difficult to remove with conventional biological treatment [2].

## 3. The Challenge of Color and AOX Removal

Color is often due to chromophoric groups in lignin, such as quinones and phenolic compounds. AOX includes chlorinated phenols, dioxins, and other halogenated organics formed during chlorine-based bleaching. These compounds resist biodegradation and are potentially carcinogenic or toxic to aquatic life [3].

Therefore, the treatment strategy must focus on:

- Degradation of high molecular weight lignin compounds

- Oxidation of AOX to less harmful compounds

- Reduction of BOD and COD simultaneously

## 4. Treatment Strategies and Neutralizing Agents

### 4.1 Aluminum Sulfate (Alum)

Function: Coagulation/flocculation
Reaction: Alum destabilizes colloids and facilitates precipitation of suspended matter.

- Advantages: Reduces turbidity, suspended solids, and part of the color.

- Disadvantages: Limited effectiveness for AOX and non-biodegradable COD.

Research: Studies show alum alone removes up to 40–50% of color, but not sufficient for AOX [4].

### 4.2 Lime (Calcium Hydroxide)

Function: pH adjustment, precipitation
Use: Often used in primary treatment.

- Advantages: Economical, removes some color and solids.

- Disadvantages: Ineffective against recalcitrant organics or AOX.

Research: Lime precipitation helps in initial COD reduction but fails to remove persistent compounds [5].

### 4.3 Hydrogen Peroxide (H₂O₂)

Function: Oxidation agent
Reaction: Breaks down chromophoric and AOX compounds via oxidative degradation.

- Advantages: Effective for color and some AOX reduction.

- Limitations: Requires high doses or catalytic assistance (e.g., UV or Fe²⁺).

Research: Peroxide alone can reduce up to 70% color, but less effective for total AOX without catalysts [6].

### 4.4 Fenton’s Reagent (H₂O₂ + Fe²⁺)

Function: Advanced oxidation
Reaction: Generates hydroxyl radicals (·OH) which degrade complex organics.

- Advantages: High efficiency in degrading AOX and lignin derivatives.

- Disadvantages: Requires pH control (~3–5), generates sludge from iron.

Research: Shown to reduce AOX by 60–90% and achieve 80% color removal in optimized conditions [7].

### 4.5 Ozone (O₃)

Function: Strong oxidizer
Use: Effective in tertiary treatment for bleaching and organics breakdown.

- Advantages: No residual chemical load; high oxidation power.

- Disadvantages: High cost, gas handling requirements.

Research: Ozonation can reduce color by 90% and significantly degrade AOX with proper retention time [8].

## 5. Comparative Analysis of Agents

| Agent | Color Removal | AOX Reduction | Cost | Byproducts/Issues |
| --- | --- | --- | --- | --- |
| Alum | Moderate | Low | Low | Sludge generation |
| Lime | Low | Low | Very Low | Requires post-neutralization |
| H₂O₂ | High (with AOP) | Moderate | Moderate | Needs catalyst for best results |
| Fenton’s | Very High | High | Moderate | Sludge, acidic pH needed |
| Ozone | Very High | High | High | High energy cost |

## 6. Recommended Approach

For modern paper mills aiming to meet stringent discharge limits, a hybrid treatment system is recommended with a combination of clarifiers, air strippers, bio reactors, ozonization and other technologies. A combination of Fenton’s reagent followed by ozonation or UV/H₂O₂ treatment provides the best results for removing color, AOX, and reducing COD. Primary and secondary treatments (e.g., sedimentation + biological processes) should be supplemented with AOPs to handle refractory contaminants.

## 7. Conclusion

Effluent treatment in paper mills must evolve to address persistent pollutants such as color and AOX. Traditional agents like alum and lime are insufficient when used alone. Advanced oxidation processes—especially those involving hydrogen peroxide, ozone, or Fenton’s reagent—show the highest potential for complete degradation of difficult contaminants. While costs are higher, the environmental benefits and compliance assurance outweigh operational expenses.

Future studies should focus on integrating AOPs with biological systems and recovering energy or materials from treated effluent to move toward zero-liquid-discharge (ZLD) systems.

## References

- Pokhrel, D., & Viraraghavan, T. (2004). Treatment of pulp and paper mill wastewater—a review. Science of the Total Environment, 333(1-3), 37–58.

- Ali, M., & Sreekrishnan, T. R. (2001). Aquatic toxicity from pulp and paper mill effluents: a review. Advances in Environmental Research, 5(2), 175–196.

- Thompson, G., Swain, J., Kay, M., & Forster, C. F. (2001). The treatment of pulp and paper mill effluent: a review. Bioresource Technology, 77(3), 275–286.

- Bajpai, P. (2015). Management of Pulp and Paper Mill Waste. Springer.

- Nurdan B., & Aysel K. (2012). Preliminary treatment of paper mill wastewater using lime and alum. Desalination and Water Treatment, 46(1-3), 157–165.

- Rana, V. S., et al. (2008). Decolorization of kraft bleach plant effluent using hydrogen peroxide. Journal of Hazardous Materials, 160(2-3), 408–415.

- Kang, S. F., Liao, C. H., & Chen, M. C. (2002). Pre-oxidation and coagulation of textile wastewater by Fenton’s reagent. Chemosphere, 46(6), 923–928.

- Ali, M., Sreekrishnan, T. R., & Gokhale, S. (1999). Ozonation of wastewater from pulp and paper industry: a pilot-scale study. Water Science and Technology, 40(11-12), 65–71.